MIRI

IICIe Daughter Board

Design Specification

IICie — I²C Extender + extensible peripheral framework

Build Flag: -D USERMOD_MIRI_BOARD_IICIE

Standalone: YES — compiles and runs without MiriCore or any other Miri mod

Hardware: Miri v1.0 · ESP32-MINI-1-H4 · IICIe v1.0 · PCA9849PWJ

Schematic: EasyEDA 2026-06-08, Avery Finchum / Finchum-Tech

Repository: https://github.com/Finchum-Tech/WLED_Miri (UWLC Project)

Framework Ref: See MiriCore Design Specification for build system details

📌  Developer Instruction: Preserve all base WLED code from the fork. Ignore Miri-specific code in the Miri_v1.0 branch — especially usermods/usermod_v2_pca9632 and platformio.ini line 352. These are reference material only. Replace Miri-specific implementations entirely with code following this specification. Do not build on top of existing Miri mods: write from scratch using this design as your source of truth.

# Framework Quick Reference

All Miri usermods share a common framework. See MiriCore Design Specification for full details. This section is a concise reference only — resolve any conflicts by deferring to MiriCore spec.

## Repository

GitHub: https://github.com/Finchum-Tech/WLED_Miri — The official WLED fork for the UWLC (Universal Wireless Lighting Controller) project. Use existing Miri code as reference only; ensure complete compatibility with WLED, overwrite with your implementation.

## miri_pins.h — Pin Abstraction

All GPIOs are defined in usermods/miri/miri_pins.h with #ifndef guards. Modules use abstract names (MIRI_I2C_SDA, MIRI_PIN_ADC_FUSE, etc.), never raw numbers. The Miri PlatformIO env -includes this file globally. Standalone users pass -D overrides.

## I²C Initialisation — Self-Init Pattern

Each mod initialises its own I²C bus. MiriCore is a bus broker, not an owner. No mod assumes another has called Wire.begin(). The ESP32 Arduino Wire library silently ignores duplicate begin() calls on the same pins, so multiple mods calling Wire.begin() with the same pins is safe — first caller wins, the rest are no-ops.

// Every mod that uses I²C calls this unconditionally in setup():

Wire.begin(MIRI_I2C_SDA, MIRI_I2C_SCL);   // main bus

// or for LP bus:

Wire1.begin(MIRI_LP_I2C_SDA, MIRI_LP_I2C_SCL, 100000);

## MIRI_DEBUG — Serial Logging

All Serial output is gated behind MIRI_DEBUG to eliminate serial overhead in production builds. Use this pattern for every debug print:

#ifdef MIRI_DEBUG

Serial.printf("[MiriXxx] reading temp: %.2f C\n", temp);

#endif

The MIRI_DEBUG flag is defined in miri_env.ini (commented out by default). Enable only during development. Do not use Serial.print() directly — ever.

## MiriState — Shared Data Bus

usermods/miri/miri_shared.h defines a lightweight struct. Each mod writes its own slice. No mod calls into another mod's C++ functions. If standalone (no other Miri mods), MiriState fields from other mods are simply not populated — code defensively.

## Web Assets — Overwrite Strategy

Miri ships a complete replacement of WLED's web assets (data/ directory) via data_dir = usermods/miri/data in miri_env.ini. Miri's index.htm is a maintained fork of WLED's — the diff is small (miri-panels div, miri-ui.js script tag, color picker replacement). Each WLED upgrade requires merging that known diff into the new version's index.htm. Never use runtime DOM injection; it is fragile.

## Drawer Architecture

Each mod that needs a main-page drawer registers a GET /miri/panel/[modname] HTTP route serving an HTML fragment. The miri-ui.js (served by MiriCore) fetches these fragments at page load and inserts them as accordion panels in the miri-panels div. Each drawer fragment is self-contained HTML/CSS/JS that talks only to WLED's /json/state and /json/info APIs.

⚠  Upstream Compliance: Before merging a new WLED tag, compile-test the Miri env. Watch for: UsermodManager lookup changes, AsyncWebServer scope changes, LEDC API changes, and index.htm structural changes that need re-merging.

# 1. Purpose & Scope

The IICIe (Pixi²) is a Miri daughter board that multiplexes up to four downstream I²C buses behind a single LP bus address using the NXP PCA9849PWJ. Up to 9 IICIe boards can coexist on the LP bus, each configured to a unique address via triple-position onboard slide switches (SW31, SW32). Each downstream channel exposes QWIIC and Stemma QT connectors for standard I²C peripherals.

IICIe is also an extensible peripheral framework. Users can easily add their own I²C peripherals (knobs, sliders, button panels, displays, etc.) by copy-pasting and modifying template code snippets. The drawer architecture is designed to make peripheral integration trivial.

📌  Board Schematic: Pixi²/IICIe - I²C Support Board, EasyEDA, created 2025-06-24, updated 2025-06-30. Available in the Miri repository at https://github.com/Finchum-Tech/WLED_Miri

# 2. Hardware

| Designator | Part | Notes |
| --- | --- | --- |
| U31 | PCA9849PWJ (NXP) | 4-channel I²C multiplexer. 2 address pins (A0, A1), each with 3 selectable positions. 4 downstream channel pairs (SD0/SC0 – SD3/SC3). Active-low RESET. TSSOP-16. Datasheet: nxp.com/docs/en/data-sheet/PCA9849.pdf |
| SW31 | MSK13C02-SZ (triple-pole single-throw) | A0 address select. Three positions: SCL, LOW (0), HIGH (1). Connects to PCA9849 pin 2. |
| SW32 | MSK13C02-SZ (triple-pole single-throw) | A1 address select. Three positions: SCL, LOW (0), HIGH (1). Connects to PCA9849 pin 13. |
| R31, R32, R34 | 10kΩ 0603 | Pull-up/pull-down resistors for address select circuits. |
| Q33 | BSS138PS (dual N-ch MOSFET) | Open-drain I²C level shift for Stemma 5V connector (CN33). Enables 5V I²C devices on a 3.3V bus using standard BSS138 topology. |
| C31, C32, C33 | 10µF ceramic | Decoupling capacitors for VDD and connector power. |
| H31 | PZ127VS-12-12-H16-4.5-R (12-pin) | Support board expansion connector — chains to the next IICIe board. |
| H32 | PM127VS-12-12-H34-4.5-R (12-pin) | Main board connector — mates with Miri H7. |
| H33 | PZ254R-11-02P (2-pin) | Additional I²C/QWIIC access header. |
| CN31, CN32 | SM04B-SRSS-TB (JST SH 1.0mm) | QWIIC connectors (3.3V I²C only). Standard QWIIC pinout: 1=GND, 2=3.3V, 3=SDA, 4=SCL. |
| CN33 | 2.0-4P WT (JST PH 2.0mm) | Stemma QT connector. 5V I²C via Q33 level shift. Standard Stemma QT pinout. |

# 3. PCA9849 I²C Address Space

## 3.1 All 9 Selectable Addresses

Each switch (A0, A1) has three selectable positions: SCL, LOW (0), or HIGH (1). This gives 3×3 = 9 total combinations. All 9 addresses are user-selectable via physical switch positions.

| A1 Position | A0 Position | 8-bit I²C addr | 7-bit addr | Range |
| --- | --- | --- | --- | --- |
| SCL | SCL | 0xB0 | 0x58 | Extended (0x58–0x5F) |
| SCL | 0 | 0xB2 | 0x59 | Extended |
| SCL | 1 | 0xB6 | 0x5B | Extended |
| 0 | SCL | 0xE0 | 0x70 | Standard (0x70–0x77) |
| 0 | 0 | 0xE2 | 0x71 | Standard |
| 0 | 1 | 0xE6 | 0x73 | Standard |
| 1 | SCL | 0xE8 | 0x74 | Standard |
| 1 | 0 | 0xEA | 0x75 | Standard |
| 1 | 1 | 0xEE | 0x77 | Standard |

Registry entry (all 9 addresses available):

static const uint8_t IICIE_ADDRS[] = {

0x58, 0x59, 0x5B,  // A1=SCL positions

0x70, 0x71, 0x73,  // A1=0 positions

0x74, 0x75, 0x77   // A1=1 positions

};

static const MiriBoardDef MIRI_BOARD_REGISTRY[] = {

{ IICIE_ADDRS, 9, MIRI_BOARD_IICIE, "IICIe" },

};

⚠  Address Conflict Avoidance: 0x70–0x77 overlaps with TCA9548A I²C multiplexers and some common devices. 0x58–0x5F is less congested. The non-consecutive spread is inherent to the PCA9849 address scheme. Peripheral device selection should account for this address map.

# 4. PCA9849 Control Register

After addressing the PCA9849, write 1 byte to select a downstream channel. Only bits B2:B1:B0 are used.

| B2:B1:B0 | Selected channel | Downstream pair |
| --- | --- | --- |
| 000 | None (de-selected) | All SD/SC pairs disconnected — default at power-on |
| 001 | Channel 0 | SD0 / SC0 |
| 010 | Channel 1 | SD1 / SC1 |
| 011 | Channel 2 | SD2 / SC2 |
| 100 | Channel 3 | SD3 / SC3 |
| 101–111 | Reserved | Do not use |

// Select channel 0:

Wire1.beginTransmission(iicieAddr);

Wire1.write(0x01);  // B2:B1:B0 = 001

Wire1.endTransmission();

# 5. QWIIC and Stemma Connectors

| Connector | Part | Standard | Voltage | Notes |
| --- | --- | --- | --- | --- |
| CN31 | SM04B-SRSS-TB (JST SH 1.0mm) | QWIIC | 3.3V | Standard pinout: 1=GND, 2=3.3V, 3=SDA, 4=SCL |
| CN32 | SM04B-SRSS-TB (JST SH 1.0mm) | QWIIC | 3.3V | Second QWIIC port — parallel to CN31 on same downstream channel |
| CN33 | 2.0-4P WT (JST PH 2.0mm) | Stemma QT | 3.3V + 5V | 5V I²C via Q33 level shift. Standard Stemma QT pinout. |
| H33 | PZ254R-11-02P (2.54mm header) | 2.54mm pitch | 3.3V | Additional I²C access (3-SCL1/3-SDA1 nets) |

The BSS138PS (Q33) provides bidirectional open-drain level shifting between 3.3V (LP bus) and 5V (Stemma device side). Pull-up resistors on both rails. Standard BSS138 topology — refer to NXP AN10441 for circuit explanation.

# 6. Extensible Peripheral Framework

## 6.1 Philosophy

IICIe is designed to make it trivial for users to add their own I²C peripherals. Rather than building peripherals directly into the main mod code, users declare and configure peripherals in a dedicated code section using predefined templates. The drawer automatically discovers, initializes, and displays all declared peripherals.

## 6.2 Peripheral Template Pattern

Each peripheral type has a copy-paste-friendly template. Users modify only the configuration — I2C address, JSON key, control range — and the framework handles the rest.

### Template 1: Analog Control (Rotary Knob, Fader Slider, etc.)

// COPY-PASTE TEMPLATE: Analog Peripheral

// Change: ADDR, JSON_KEY, MIN_VALUE, MAX_VALUE, LABEL

struct AnalogPeripheral {

uint8_t i2cAddr;

const char* jsonKey;   // e.g. "r", "g", "b", "brightness"

uint8_t minVal, maxVal;

const char* label;

uint8_t lastVal;

unsigned long lastRead;

};

// Configuration:

static const AnalogPeripheral PERIPH_4KNOB = {

.i2cAddr = 0x42,          // CHANGE: I2C address of knob controller

.jsonKey = "r",          // CHANGE: "r", "g", "b", custom key, etc.

.minVal  = 0,

.maxVal  = 255,

.label   = "Red Knob",    // CHANGE: display name

};

// Framework handles: polling, JSON POST, UI rendering

### Template 2: Digital Control (Button Panel, Encoder)

// COPY-PASTE TEMPLATE: Digital Peripheral

// Change: ADDR, NUM_BUTTONS, ACTIONS

struct DigitalPeripheral {

uint8_t i2cAddr;

uint8_t numButtons;

const char* actions[8];   // CHANGE: effect names, "next", "prev", etc.

const char* labels[8];    // CHANGE: button labels

uint8_t lastState;

};

// Configuration:

static const DigitalPeripheral PERIPH_BUTTONS = {

.i2cAddr    = 0x44,

.numButtons = 4,

.actions    = { "next_effect", "prev_effect", "next_pal", "prev_pal" },

.labels     = { "Eff↓", "Eff↑", "Pal↓", "Pal↑" },

};

### Template 3: Display Peripheral (Text/Icon Status)

// COPY-PASTE TEMPLATE: Display Peripheral

// Change: ADDR, JSON_SOURCE, UPDATE_INTERVAL

struct DisplayPeripheral {

uint8_t i2cAddr;

const char* jsonSource;  // e.g. "effect", "palette", custom key

uint16_t updateIntervalMs;

char lastText[32];

};

// Configuration:

static const DisplayPeripheral PERIPH_DISPLAY = {

.i2cAddr            = 0x46,

.jsonSource         = "effect",  // CHANGE: which JSON field to display

.updateIntervalMs   = 500,

};

## 6.3 Registering User Peripherals

In the IICIe driver's activate() function, peripherals are registered in an array:

// User Peripheral Registration

static const AnalogPeripheral* analogPeripherals[] = {

&PERIPH_4KNOB,

// ADD YOUR ANALOG PERIPHERALS HERE

};

static const uint8_t numAnalogPeripherals =

sizeof(analogPeripherals) / sizeof(analogPeripherals[0]);

// Framework iterates, polls, and updates each one.

## 6.4 Drawer Auto-Generation

The drawer HTML is auto-generated from the peripheral declarations. For each analog peripheral: a label, live value display (0–255), and a slider. For each digital peripheral: buttons. The drawer sends POST requests to /json/state with the appropriate key/value pairs.

// Generated drawer fragment (pseudocode):

<div class="peripheral-analog">

<label>Red Knob</label>

<input type="range" min="0" max="255" value="128">

<span>128</span>

<button onclick="post_json_state('r', value)">Apply</button>

</div>

# 7. Per-Instance Firmware Driver

## 7.1 Activation

MiriDaughterboards detects each IICIe instance at its 7-bit address. For each match, instantiates MiriBoard_IICIe(addr) and calls activate(addr, wire1_handle). The driver registers its HTTP drawer route at /miri/panel/iicie/0x[addr].

## 7.2 Channel Selection & Downstream Scan

The drawer shows a scan channel button, the mod writes the PCA9849 control register to select that channel. The button triggers a fresh I²C scan on the IICie hardwired downstream (channel 0 on the chip) and returns a list of detected addresses. Scan for downstream components is not automatic at boot: it is only manual through the interface or on user code request.

## 7.3 User Code Execution Area

The IICIe mod includes a designated section at the top (or in a separate included file) where users copy-paste peripheral templates and declare their instances. Comments guide users through configuration. Example:

// ═══════════════════════════════════════════════════════════

// USER PERIPHERAL DECLARATIONS — COPY-PASTE TEMPLATES HERE

// ═══════════════════════════════════════════════════════════

// Example 1: 4-position rotary encoder for brightness

static const AnalogPeripheral PERIPH_BRIGHTNESS = {

.i2cAddr = 0x42,          // ← CHANGE: your encoder's I2C address

.jsonKey = "bri",        // ← CHANGE: control JSON key

.minVal  = 0, .maxVal = 255,

.label   = "Brightness",

};

// Example 2: Button panel for playlist control

static const DigitalPeripheral PERIPH_PLAYLIST = {

.i2cAddr    = 0x44,       // ← CHANGE: your button board address

.numButtons = 3,		 //  reads consecutive bits

.actions    = { "prev_preset", "play", "next_preset" },

.labels     = { "|◄", "►", "►|" },

};

// Register above in activate():

static const AnalogPeripheral* analogPeripherals[] = {

&PERIPH_BRIGHTNESS,

};

# 8. WLED API Hooks (per instance)

| WLED Hook | Implementation Detail |
| --- | --- |
| activate(addr, wire) | Called when address detected. Store address & wire. Register GET /miri/panel/iicie/0x[addr]. Init all declared user peripherals. |
| loop() | Poll analog peripherals every pollIntervalMs. Read current value via I²C on selected channel. If changed: POST /json/state with jsonKey and value. |
| GET /miri/panel/iicie/0x[addr] | Serve auto-generated drawer HTML. Show channel selector, scan button, and all peripheral controls. |
| POST /miri/iicie/0x[addr]/channel | Set PCA9849 control register to select a channel (0–3). |
| POST /miri/iicie/0x[addr]/scan | Scan selected downstream channel; return detected I²C addresses as JSON. |

# 9. Error Handling

| Failure | Detection | Response |
| --- | --- | --- |
| PCA9849 not found | endTransmission() != 0 at boot | Remove from active list. Log (MIRI_DEBUG). |
| Downstream device NAK | I2C read times out on selected channel | Show error badge in drawer. Retry on next poll. |
| Peripheral read stale | No update for > 2 × pollIntervalMs | Gray out drawer control. Continue polling. |
| Channel switch fails | Write control register fails | Log error. Retry channel selection. |

# 10. Resolved Questions

✓  I²C Address Range: All 9 user-selectable addresses confirmed: 0x58, 0x59, 0x5B, 0x70, 0x71, 0x73, 0x74, 0x75, 0x77. All 9 are available in the registry. Users set switches for their desired address.

✓  Downstream Scan Trigger: Manual/on-request only. User clicks 'Scan' button in drawer. Scan runs on currently-selected channel. No automatic boot scan.

✓  RESET# Coupling: RESET# is physically tied to main board RST net with no isolation. IICIe resets when main board resets. No dedicated GPIO to decouple. This is hardware limitation; no firmware workaround.

✓  Stemma 5V Rail: 5V rail is physically selectable only — user hardware configuration (jumper/switch). Not software-controlled. Always-on when hardware is jumpered.

# 11. Datasheet References

- PCA9849: nxp.com/docs/en/data-sheet/PCA9849.pdf — §6.1 (address table), §6.3 (control register).

- BSS138PS level shift: NXP AN10441 — bidirectional I²C level-shift topology.

- QWIIC standard: sparkfun.com/qwiic — connector pinout.

- Stemma QT: adafruit.com/stemma — pinout and electrical specification.

- MSK13C02-SZ: supplier datasheet — triple-position slide switch characteristics.