MIRI

MiriCore

Design Specification

Integration layer, branding, alert management, bus broker

Build Flag: -D USERMOD_MIRI

Standalone: NO- calls all miri components

Hardware: Miri v1.0 · ESP32-MINI-1-H4

Schematic: EasyEDA 2026-06-08, Avery Finchum / Finchum-Tech

Repository: https://github.com/Finchum-Tech/WLED_Miri (UWLC Project)

Framework Ref: See MiriCore Design Specification for build system details

📌  Developer Instruction: Preserve all base WLED code from the fork. Ignore Miri-specific code in the Miri_v1.0 branch — especially usermods/usermod_v2_pca9632 and platformio.ini line 352. These are reference material only. Replace Miri-specific implementations entirely with code following this specification. Do not build on top of existing Miri mods: write from scratch using this design as your source of truth.

# ## Build Configuration & Sub-Mod Bundling

**When MiriCore is enabled, all Miri sub-mods are automatically included.** This is a build-time bundling mechanism handled via PlatformIO's `extra_configs` system.

### How It Works

**platformio.ini** (top level):

```ini

[platformio]

extra_configs =

platformio_override.ini

usermods/miri/miri_env.ini

```

**usermods/miri/miri_env.ini** (defines the Miri bundle):

```ini

[env:miri_esp32]

extends = esp32

build_flags = ${env.build_flags}

-D USERMOD_MIRI                      # Core

-D USERMOD_MIRI_TEMP_SENSOR          # Bundle

-D USERMOD_MIRI_PWM                  # Bundle

-D USERMOD_MIRI_FUSE_MONITOR         # Bundle

-D USERMOD_MIRI_DAUGHTERBOARDS       # Bundle

-D USERMOD_MIRI_COLOR_SPHERE         # Bundle

-include usermods/miri/miri_pins.h

```

When a developer builds with `-e miri_esp32`, all seven Miri modules compile and link together into a single firmware image.

### Standalone Use

Individual sub-mods can be used **without MiriCore** on any WLED-capable hardware. A developer who only wants MiriTempSensor can add `-D USERMOD_MIRI_TEMP_SENSOR` to their own environment in platformio.ini without touching the Miri bundle. The sub-mod will compile independently with zero MiriCore overhead.

### Maintenance

The bundled flag list in `miri_env.ini` is the single source of truth for which sub-mods ship with Miri. If a new sub-mod is added (e.g., a new daughter board driver), it is added to this list so all users building for `miri_esp32` automatically get it. Individual users building on non-Miri hardware are unaffected — they opt-in to only the mods they need.

# 1. Purpose & Scope

MiriCore is the integration layer. It instantiates and registers sub-mods, applies Miri branding (serverDescription + CSS/JS injection into the modified index.htm), monitors MiriState alert flags, and acts as an I²C bus broker — broadcasting which GPIO pairs map to which bus, but not owning bus init. MiriCore does not own any peripheral hardware directly.

MiriCore is NOT standalone. It is the Miri-specific glue layer. Sub-mods run without it on any WLED hardware.

📌  Repository: https://github.com/Finchum-Tech/WLED_Miri — UWLC Project. Treat existing code as reference only. Overwrite what is there with this architecture.

# 2. I²C Bus Broker Role

MiriCore does not call Wire.begin() or own bus initialisation. Each mod that uses I²C calls Wire.begin() in its own setup(). The ESP32 Wire library handles duplicate calls gracefully. MiriCore's role is purely to define — via miri_pins.h — which GPIO pairs correspond to which bus, so every mod knows what values to pass.

| Bus | SDA GPIO | SCL GPIO | Wire Object | Devices |
| --- | --- | --- | --- | --- |
| Main I²C | IO21 | IO22 | Wire | TMP1075 (0x49), PCA9633 (0x62) |
| LP I²C | IO2 (GPIO2) | TBD (RTC GPIO) | Wire1 | Daughter boards (scanned) |

## Serial Debug — MIRI_DEBUG

All Serial output must be wrapped in #ifdef MIRI_DEBUG. The flag is off by default in miri_env.ini. Serial overhead at production frame rates is not acceptable.

#ifdef MIRI_DEBUG

Serial.printf("[ThisMod] value: %d\n", val);

#endif

# 3. Branding

## 3.1 Brand Identity

| Element | Value |
| --- | --- |
| Primary colour | #2EB8B8 (teal — from Miri logo) |
| Logo style | Segmented/digital display font |
| serverDescription | "Miri" |
| mDNS hostname | miri-[chip-id] (confirm with product owner) |

## 3.2 Web Asset Strategy

Miri ships a complete replacement of WLED's data/ directory via data_dir = usermods/miri/data in miri_env.ini. The modified index.htm adds: a <div id="miri-panels"></div>, a <script src="/miri-ui.js"></script>, replaces the native 2D colour picker container for MiriColorSphere, and applies #2EB8B8 accent colour via inline CSS variables. This is a maintained diff — merge it into each new WLED index.htm on version upgrades.

## 3.3 miri-ui.js

Served at /miri-ui.js. Fetches /miri/panel/[modname] from each active mod and inserts accordion panels into #miri-panels. Panels register themselves at runtime — adding a new mod never requires touching index.htm again.

# 4. Sub-Mod Registration Order

| Order | Mod | Reason |
| --- | --- | --- |
| 1 | MiriTempSensor | Read first; overTemp flag must be live before alert loop runs. |
| 2 | MiriFuseMonitor | ADC only; no I²C dependency. |
| 3 | MiriPWM | LEDC/GPIO; init after power confirmed healthy. |
| 4 | MiriDaughterboards | LP bus scan; runs after main bus confirmed. |
| 5 | MiriColorSphere | HTTP route only; web server must be ready. |

# 5. Alert Loop

In loop(), check MiriState.overTemp and MiriState.fuseBlown on a rate-limited basis (alertIntervalMs). On rising edge: LED notification (segment 0 red blink for alertDurationMs) and/or MQTT publish to alertMqttTopic. Log to Serial inside #ifdef MIRI_DEBUG.

# 6. Planned Daughter Board Ecosystem

The following daughter boards are in the Miri pipeline. Each detected instance gets its own drawer in the WLED index page via the miri-ui.js panel system.

| Board | Main Page Drawer | Notes |
| --- | --- | --- |
| IICIe (Pixi²) | Yes — per instance | I²C multiplexer. Up to 6 on LP bus simultaneously. |
| Audio Player | Yes | Dedicated audio playback drawer. |
| Microphone | Yes | Live audio input / VU display. |
| Touch Sense Expander | Yes | Touch pad status and sensitivity. |
| Color Spectrum Analyzer | Yes | Live frequency spectrum display. |
| Ethernet | None | Compile-time WLED option (-D WLED_USE_ETHERNET). No mod needed — use a supported chip and enable via build flag. |

# 7. WLED API Hooks

| WLED Hook | Implementation Detail |
| --- | --- |
| setup() | Set serverDescription="Miri". Register GET /miri-ui.js and GET /miri-brand.css. Instantiate sub-mods in order (§4). Set MiriState.systemReady=true. Log active mods to Serial (MIRI_DEBUG only). |
| loop() | Rate-limited alert check (alertIntervalMs). On rising edge of overTemp or fuseBlown: trigger alert action per alertMode config. |
| addToJsonInfo() | Emit: system name, active sub-mod list, current alert flags, firmware build date. |
| addToConfig() | alertIntervalMs, alertDurationMs, alertMode (0=LED/1=MQTT/2=both), alertMqttTopic. |
| readFromConfig() | Defaults: alertIntervalMs=5000, alertDurationMs=3000, alertMode=0, alertMqttTopic="miri/alert". |

# 8. Configuration Reference

| Config Key | Type | Default | Valid Range | Description |
| --- | --- | --- | --- | --- |
| alertIntervalMs | uint32 | 5000 | 1000–60000 | Minimum ms between alert triggers. |
| alertDurationMs | uint32 | 3000 | 500–10000 | Duration of LED blink alert. |
| alertMode | uint8 | 0 | 0,1,2 | 0=LED only, 1=MQTT only, 2=both. |
| alertMqttTopic | string | miri/alert | — | MQTT topic for alert publish. |

h1("X. Code Minimalism & Flash Budget")

p("Miri runs on an ESP32-MINI-1-H4 with limited flash (~4MB flash chip with 2MB available total for code + data + WLED due to OTA). Multiple usermods compile into a single firmware image. Each module must be ruthlessly minimal.")

bullet("No redundant code. Every line must earn its space.", 0)

bullet("Loop time: keep loop() under 1ms. Offload heavy work to FreeRTOS tasks.", 0)

bullet("Use const strings in PROGMEM for static text (HTML fragments, labels, debug strings).", 0)

bullet("Avoid large buffers or arrays unless essential. Reuse buffers where possible.", 0)

Bullet(“you may add to miri_shared.h(lives in miri core mod) and you are encouraged to reuse any code and strings in there.”, 0)

bullet("Share common logic via miri_shared.h. Do not duplicate code across mods.", 0)

bullet("Share common const elements such as html elements via miri_shared.h. Do not duplicate strings across mods.", 0)

bullet("Daughter board drivers compile only if their -D USERMOD_MIRI_BOARD_XXXXX flag is set. Uncompiled mods add zero overhead.", 0)

p("A well-written mod is often 50-100 lines of C++ total (setup + loop + hooks). Complex mods like MiriPWM may reach 300-400 lines because they own real hardware. Aim for clarity and compactness, not cleverness.")

warn("Build Flag Discipline","Always wrap mod code in #ifdef USERMOD_MIRI_XXXXX / #endif. If a mod is not enabled at build time, it contributes nothing to the final image — no linker overhead.")

# 9. Open Questions

❓  Open Question: Brand logo SVG and final CSS variables needed before branding implementation.

❓  Open Question: Should mDNS hostname be prefixed 'miri-' or left to user config?

❓  Open Question: Who owns PCA9633 (U11) — MiriCore (status LEDs) or MiriPWM (PWM output)?