MIRI

MiriPWM

Design Specification

RGBW 97kHz PWM 6 Amp MOSFET controlled LED strip driver

Build Flag: -D USERMOD_MIRI_PWM

Standalone: YES — compiles and runs without MiriCore or any other Miri mod

Hardware: Miri v1.0 · ESP32-MINI-1-H4

Schematic: EasyEDA 2026-06-08, Avery Finchum / Finchum-Tech

Repository: https://github.com/Finchum-Tech/WLED_Miri (UWLC Project)

Framework Ref: See MiriCore Design Specification for build system details

📌  Developer Instruction: Preserve all base WLED code from the fork. Ignore Miri-specific code in the Miri_v1.0 branch — especially usermods/usermod_v2_pca9632 and platformio.ini line 352. These are reference material only. Replace Miri-specific implementations entirely with code following this specification. Do not build on top of existing Miri mods: write from scratch using this design as your source of truth.

# Framework Quick Reference

All Miri usermods share a common framework. See MiriCore Design Specification for full details. This section is a concise reference only — resolve any conflicts by deferring to MiriCore spec.

## Repository

GitHub: https://github.com/Finchum-Tech/WLED_Miri — The official WLED fork for the UWLC (Universal Wireless Lighting Controller) project. Use existing code as reference only; overwrite with your implementation.

## miri_pins.h — Pin Abstraction

All GPIOs are defined in usermods/miri/miri_pins.h with #ifndef guards. Modules use abstract names (MIRI_I2C_SDA, MIRI_PIN_ADC_FUSE, etc.), never raw numbers. The Miri PlatformIO env -includes this file globally. Standalone users pass -D overrides.

## I²C Initialisation — Self-Init Pattern

Each mod initialises its own I²C bus. MiriCore is a bus broker, not an owner. No mod assumes another has called Wire.begin(). The ESP32 Arduino Wire library silently ignores duplicate begin() calls on the same pins, so multiple mods calling Wire.begin() with the same pins is safe — first caller wins, the rest are no-ops.

// Every mod that uses I²C calls this unconditionally in setup():

Wire.begin(MIRI_I2C_SDA, MIRI_I2C_SCL);   // main bus

// or for LP bus:

Wire1.begin(MIRI_LP_I2C_SDA, MIRI_LP_I2C_SCL, 100000);

## MIRI_DEBUG — Serial Logging

All Serial output is gated behind MIRI_DEBUG to eliminate serial overhead in production builds. Use this pattern for every debug print:

#ifdef MIRI_DEBUG

Serial.printf("[MiriXxx] reading temp: %.2f C\n", temp);

#endif

The MIRI_DEBUG flag is defined in miri_env.ini (commented out by default). Enable only during development. Do not use Serial.print() directly — ever.

## MiriState — Shared Data Bus

usermods/miri/miri_shared.h defines a lightweight struct. Each mod writes its own slice. No mod calls into another mod's C++ functions. If standalone (no other Miri mods), MiriState fields from other mods are simply not populated — code defensively.

## Web Assets — Overwrite Strategy

Miri ships a complete replacement of WLED's web assets (data/ directory) via data_dir = usermods/miri/data in miri_env.ini. Miri's index.htm is a maintained fork of WLED's — the diff is small (miri-panels div, miri-ui.js script tag, color picker replacement). Each WLED upgrade requires merging that known diff into the new version's index.htm. Never use runtime DOM injection; it is fragile.

## Drawer Architecture

Each mod that needs a main-page drawer registers a GET /miri/panel/[modname] HTTP route serving an HTML fragment. The miri-ui.js (served by MiriCore) fetches these fragments at page load and inserts them as accordion panels in the miri-panels div. Each drawer fragment is self-contained HTML/CSS/JS that talks only to WLED's /json/state and /json/info APIs.

⚠  Upstream Compliance: Before merging a new WLED tag, compile-test the Miri env. Watch for: UsermodManager lookup changes, AsyncWebServer scope changes, LEDC API changes, and index.htm structural changes that need re-merging.

# 1. Purpose & Scope

MiriPWM controls all LED output channels on the Miri v1.0 board: four raw RGBW PWM channels (N-channel MOSFET pairs) and two addressable LED strip channels (via the SN74HCS244 5V level-shift buffer). It also owns CH_EN and the PCA9633 I²C indicator driver.

V1 scope: make RGBW PWM and strip outputs work reliably. No main-page drawer. No settings drawer in v1.

📌  V2 Placeholder: A settings-page dropdown drawer for channel-to-colour assignment and colour temperature mixing is planned for v2. Spec and reserve the settings section now — implement in the next revision.

✓  Q5 DMP3050: P-channel MOSFET providing reverse polarity protection for the 3.3V logic rail only. Passive component — no firmware involvement. The power/LED side (PWRIN) has TVS transient protection only, no reverse polarity protection.

✓  CH_EN Function: External reset/enable line. Asserted HIGH by firmware on normal boot to enable SN74HCS244 outputs. External hardware (programmer or daughter board) can pull it LOW to disable all strip/PWM outputs safely and optionally reset the ESP32 for UART programming or low-power wake cycles. Routed to daughter board connector H7.

# 2. Hardware

| Designator | Part | Notes |
| --- | --- | --- |
| Q1–Q4 | TSM900N06CP ROG (N-ch, US variant) | RGBW PWM. Gate drive: verify Vgs(th) max ≤ 2.5V — driven by 3.3V GPIO. Datasheet from supplier. |
| Q6–Q9 | DOD30N06 (N-ch, Asia variant) | Parallel with Q1–Q4 for higher current. Same gate signal per channel. |
| Q5 | DMP3050 (P-ch MOSFET) | Logic-side reverse polarity protection. Passive — no firmware control. |
| U5 | SN74HCS244RKSR (TI Octal Buffer) | 3.3V→5V level shift for LED strip data/clock. VCC=+5V (from U2). OE# = CH_EN. Datasheet: ti.com/product/SN74HCS244 |
| U11 | PCA9633DP1,118 (NXP) | I²C 4-ch LED driver at 0x62. Drives onboard indicator LEDs. Datasheet: nxp.com/products/PCA9633 |
| U2 | TPS613222ADBVT (TI Boost) | Generates +5V rail for U5 and strip power pads. Datasheet: ti.com/product/TPS61322 |

# 3. Output Channel Map

| Channel | Net | GPIO | Path | Connector |
| --- | --- | --- | --- | --- |
| LED_R | LED_R | IO13 | Direct → Q1+Q9 gates | H1 (raw PWM pads) |
| LED_G | LED_G | IO12 | Direct → Q2+Q6 gates | H1 |
| LED_B | LED_B | IO14 | Direct → Q3+Q7 gates | H1 |
| LED_W | LED_W | IO27 | Direct → Q4+Q8 gates | H1 |
| Strip1 Data | 1PINSTRIPDATA | IO13 | → U5 2A1 → H2/H3 | Neopixel pads |
| Strip1 Clock | 1PINSTRIPCLOCK | IO15 | → U5 2A4 → H4 | 4-pin strip pad |
| Strip2 Data | 2PINSTRIPDATA | IO14 | → U5 2A3 → H3/H5 | Neopixel pads |
| Strip2 Clock | 2PINSTRIPCLOCK | IO12 | → U5 2A2 → H5 | 4-pin strip pad |

⚠  Shared GPIO — Mutual Exclusion: IO12, IO13, IO14 are each shared between MOSFET gate drive (PWM mode) and SN74HCS244 inputs (strip mode). A given GPIO cannot do both at once. Enforce a per-channel mode config (PWM vs strip) and prevent both from being active on the same pin.

⚠  GPIO12 Strapping (MTDI): Must be LOW at boot on ESP32-MINI-1-H4 (3.3V flash). Confirm U5 input holds GPIO12 low when OE# is asserted during boot. Ref: ESP32-MINI-1 datasheet §2.4.

⚠  GPIO15 Strapping (MTDO): Must be LOW or floating at boot. Confirm pull-down or U5 holds it. Same datasheet reference.

# 4. Pin Assignments

## I²C Initialisation — Standalone Pattern

This mod calls Wire.begin() unconditionally in setup(). MiriCore does not own I²C init. If MiriCore is present, its call already ran and this one is a no-op — the ESP32 Wire library silently ignores duplicate begin() calls on the same pins.

Wire.begin(MIRI_I2C_SDA, MIRI_I2C_SCL);

Wire.setClock(400000); // 400 kHz Fast Mode default

| Signal / Net | ESP32 GPIO | Direction | Notes |
| --- | --- | --- | --- |
| LED_R gate | IO13 | Output (LEDC) | Q1+Q9. Verify 3.3V Vgs(th) drives both MOSFETs into saturation. |
| LED_G gate | IO12 | Output (LEDC) | Q2+Q6. ⚠ Strapping pin. |
| LED_B gate | IO14 | Output (LEDC) | Q3+Q7. Also JTAG TMS. |
| LED_W gate | IO27 | Output (LEDC) | Q4+Q8. No strapping conflict. |
| 1PINSTRIPDATA | IO13 | Output | Via U5. Shared with LED_R — mutually exclusive. |
| 1PINSTRIPCLOCK | IO15 | Output | Via U5. ⚠ Strapping pin. |
| 2PINSTRIPDATA | IO14 | Output | Via U5. Shared with LED_B — mutually exclusive. |
| 2PINSTRIPCLOCK | IO12 | Output | Via U5. Shared with LED_G — mutually exclusive. |
| CH_EN | TBD | Output | U5 OE# (active low). Also on H7 daughter board connector. Confirm GPIO with designer. |
| SDA / SCL | IO21 / IO22 | Bidir | For PCA9633 (U11). Call Wire.begin() in setup(). |

## Serial Debug — MIRI_DEBUG

All Serial output must be wrapped in #ifdef MIRI_DEBUG. The flag is off by default in miri_env.ini. Serial overhead at production frame rates is not acceptable.

#ifdef MIRI_DEBUG

Serial.printf("[ThisMod] value: %d\n", val);

#endif

# 5. LEDC Configuration

Check WLED source (bus_manager.h or similar in the pinned WLED version) for which LEDC channels WLED reserves before assigning Miri channels. Do not hardcode channel numbers without this check.

// Assign after confirming WLED-reserved channels in pinned source:

LEDC_CH_? → LED_R (IO13)

LEDC_CH_? → LED_G (IO12)

LEDC_CH_? → LED_B (IO14)

LEDC_CH_? → LED_W (IO27)

// Default frequency: 1220 Hz at 12-bit resolution

# 6. V2 Settings Placeholder

Reserve a section in addToConfig()/readFromConfig() for the v2 channel assignment feature. Store keys now (e.g. ch1ColorRole, ch2ColorRole) but leave them unused. The v2 settings drawer will expose a dropdown to reassign which driver output maps to which colour role (useful for colour temperature mixing and non-standard RGBW arrangements).

# 7. WLED API Hooks

| WLED Hook | Implementation Detail |
| --- | --- |
| setup() | Wire.begin(). Init PCA9633 (clear SLEEP bit, mode register). Configure LEDC channels per saved mode (PWM vs strip). Assert CH_EN HIGH (enable U5). Register WLED bus outputs for strip channels if strip mode active. |
| loop() | Update LEDC duty on WLED state change. Update PCA9633 on indicator state change. Write I²C only on change. Keep < 1 ms total. |
| addToJsonInfo() | Emit channel mode map, CH_EN state, v2 placeholder fields. |
| addToConfig() | ch1Mode, ch2Mode, ledcFreqHz, pca9633Brightness, (reserved: ch1ColorRole, ch2ColorRole). |
| readFromConfig() | Defaults: ch1Mode=0 (PWM), ch2Mode=0, ledcFreqHz=1220, pca9633Brightness=128. |

# 8. Open Questions

❓  Open Question: Which LEDC channels does the pinned WLED version reserve? Check source before assigning.

❓  Open Question: Which GPIO drives CH_EN? Confirm with hardware designer.

❓  Open Question: What do PCA9633 LED0–LED3 outputs drive? Confirm with designer.

❓  Open Question: Is TPS613222 EN pin shared with CH_EN? This would mean CH_EN also disables the 5V rail.