MIRI

MiriTempSensor

Design Specification

TMP1075NDRLR I²C temperature sensor

Build Flag: -D USERMOD_MIRI_TEMP_SENSOR

Standalone: YES — compiles and runs without MiriCore or any other Miri mod

Hardware: Miri v1.0 · ESP32-MINI-1-H4

Schematic: EasyEDA 2026-06-08, Avery Finchum / Finchum-Tech

Repository: https://github.com/Finchum-Tech/WLED_Miri (UWLC Project)

Framework Ref: See MiriCore Design Specification for build system details

📌  Developer Instruction: Preserve all base WLED code from the fork. Ignore Miri-specific code in the Miri_v1.0 branch — especially usermods/usermod_v2_pca9632 and platformio.ini line 352. These are reference material only. Replace Miri-specific implementations entirely with code following this specification. Do not build on top of existing Miri mods: write from scratch using this design as your source of truth.

# Framework Quick Reference

All Miri usermods share a common framework. See MiriCore Design Specification for full details. This section is a concise reference only — resolve any conflicts by deferring to MiriCore spec.

## Repository

GitHub: https://github.com/Finchum-Tech/WLED_Miri — The official WLED fork for the UWLC (Universal Wireless Lighting Controller) project. Use existing code as reference only; overwrite with your implementation.

## miri_pins.h — Pin Abstraction

All GPIOs are defined in usermods/miri/miri_pins.h with #ifndef guards. Modules use abstract names (MIRI_I2C_SDA, MIRI_PIN_ADC_FUSE, etc.), never raw numbers. The Miri PlatformIO env -includes this file globally. Standalone users pass -D overrides.

## I²C Initialisation — Self-Init Pattern

Each mod initialises its own I²C bus. MiriCore is a bus broker, not an owner. No mod assumes another has called Wire.begin(). The ESP32 Arduino Wire library silently ignores duplicate begin() calls on the same pins, so multiple mods calling Wire.begin() with the same pins is safe — first caller wins, the rest are no-ops.

// Every mod that uses I²C calls this unconditionally in setup():

Wire.begin(MIRI_I2C_SDA, MIRI_I2C_SCL);   // main bus

// or for LP bus:

Wire1.begin(MIRI_LP_I2C_SDA, MIRI_LP_I2C_SCL, 100000);

## MIRI_DEBUG — Serial Logging

All Serial output is gated behind MIRI_DEBUG to eliminate serial overhead in production builds. Use this pattern for every debug print:

#ifdef MIRI_DEBUG

Serial.printf("[MiriXxx] reading temp: %.2f C\n", temp);

#endif

The MIRI_DEBUG flag is defined in miri_env.ini (commented out by default). Enable only during development. Do not use Serial.print() directly — ever.

## MiriState — Shared Data Bus

usermods/miri/miri_shared.h defines a lightweight struct. Each mod writes its own slice. No mod calls into another mod's C++ functions. If standalone (no other Miri mods), MiriState fields from other mods are simply not populated — code defensively.

## Web Assets — Overwrite Strategy

Miri ships a complete replacement of WLED's web assets (data/ directory) via data_dir = usermods/miri/data in miri_env.ini. Miri's index.htm is a maintained fork of WLED's — the diff is small (miri-panels div, miri-ui.js script tag, color picker replacement). Each WLED upgrade requires merging that known diff into the new version's index.htm. Never use runtime DOM injection; it is fragile.

## Drawer Architecture

Each mod that needs a main-page drawer registers a GET /miri/panel/[modname] HTTP route serving an HTML fragment. The miri-ui.js (served by MiriCore) fetches these fragments at page load and inserts them as accordion panels in the miri-panels div. Each drawer fragment is self-contained HTML/CSS/JS that talks only to WLED's /json/state and /json/info APIs.

⚠  Upstream Compliance: Before merging a new WLED tag, compile-test the Miri env. Watch for: UsermodManager lookup changes, AsyncWebServer scope changes, LEDC API changes, and index.htm structural changes that need re-merging.

# 1. Purpose & Scope

Reads the onboard TI TMP1075NDRLR digital temperature sensor (U6) over the main I²C bus at 0x49. Exposes board temperature in the WLED info panel and via a main-page drawer. Writes tempCelsius and overTemp to MiriState. Does not take protective action — MiriCore handles alerts.

# 2. Hardware

| Designator | Part | Notes |
| --- | --- | --- |
| U6 | TMP1075NDRLR (TI) | I²C digital temp sensor. ±0.5°C accuracy. 12-bit, 0.0625°C/LSB. SOT-563. Datasheet: ti.com/product/TMP1075 |
| R2, R3 | 10kΩ 0603 pull-ups | SDA/SCL pull-ups to +3.3V. Already on board — no external pull-ups. |
| R5–R8 | 10kΩ 0603 | Address select resistors. A0 (pin 4) tied to +3.3V → I²C address 0x49. |

| I²C Address | A0 State | Confirmed? |
| --- | --- | --- |
| 0x49 | VCC (A0 tied to +3.3V) | YES — from schematic netlist |
| 0x48 | GND | Not this board |

# 3. Pin Assignments

## I²C Initialisation — Standalone Pattern

This mod calls Wire.begin() unconditionally in setup(). MiriCore does not own I²C init. If MiriCore is present, its call already ran and this one is a no-op — the ESP32 Wire library silently ignores duplicate begin() calls on the same pins.

Wire.begin(MIRI_I2C_SDA, MIRI_I2C_SCL);

Wire.setClock(400000); // 400 kHz Fast Mode default

| Signal / Net | ESP32 GPIO | Direction | Notes |
| --- | --- | --- | --- |
| SDA (main I²C) | IO21 | Bidirectional | Shared with PCA9633 (U11). Pull-ups R2/R3 on board. |
| SCL (main I²C) | IO22 | Output | Same bus. Wire.begin() is safe to call — duplicate calls are no-ops. |
| ALERT | N/C | (future input) | Unconnected on Miri v1.0. Reserved for interrupt-driven over-temp in future revision. |

#ifndef MIRI_I2C_SDA

#define MIRI_I2C_SDA 21

#endif

#ifndef MIRI_I2C_SCL

#define MIRI_I2C_SCL 22

#endif

## Serial Debug — MIRI_DEBUG

All Serial output must be wrapped in #ifdef MIRI_DEBUG. The flag is off by default in miri_env.ini. Serial overhead at production frame rates is not acceptable.

#ifdef MIRI_DEBUG

Serial.printf("[ThisMod] value: %d\n", val);

#endif

# 4. Register Map

| Register | Addr | Bytes | Format |
| --- | --- | --- | --- |
| Temp Register | 0x00 | 2 | Bits [15:4] = 12-bit signed, 0.0625°C/LSB. MSB first. |
| Config Register | 0x01 | 2 | Default 0x0000 = continuous, 12-bit. Write on init to confirm. |
| T_LOW | 0x02 | 2 | ALERT low threshold — not used in polling mode. |
| T_HIGH | 0x03 | 2 | ALERT high threshold — not used in polling mode. |

// Temperature decode:

int16_t raw  = (int16_t)((msb << 8) | lsb) >> 4;

float   degC = raw * 0.0625f;

# 5. MiriState Interaction

| Field | Action | Condition |
| --- | --- | --- |
| MiriState.tempCelsius | Write every successful poll | Always |
| MiriState.overTemp | Write true | tempCelsius > overTempThreshold_C |
| MiriState.overTemp | Write false | tempCelsius <= (overTempThreshold_C - 2.0) — hysteresis |

# 6. Main-Page Drawer

Register GET /miri/panel/tempsensor in setup(). The HTML fragment shows a live temperature gauge updated via fetch('/json/info'). The drawer also shows the overTemp alert badge when active. Threshold configuration stays in WLED settings (addToConfig), not in the drawer.

# 7. WLED API Hooks

| WLED Hook | Implementation Detail |
| --- | --- |
| setup() | Wire.begin(MIRI_I2C_SDA, MIRI_I2C_SCL). Write config register 0x0000. Probe 0x49 — log error and set _sensorOk=false if no ACK. Register GET /miri/panel/tempsensor. |
| loop() | Poll every pollIntervalMs. Read 2 bytes from 0x00. Decode. Write MiriState. Check threshold with hysteresis. |
| addToJsonInfo() | { "Board Temp": [value, "°C"] }. If overTemp: { "TEMP ALERT": [1, "!"] }. |
| addToConfig() | pollIntervalMs, overTempThreshold_C. |
| readFromConfig() | Defaults: pollIntervalMs=2000, overTempThreshold_C=70.0. Validate ranges. |

# 8. Configuration Reference

| Config Key | Type | Default | Valid Range | Description |
| --- | --- | --- | --- | --- |
| pollIntervalMs | uint32 | 2000 | 500–30000 | Sensor poll interval in ms. |
| overTempThreshold_C | float | 70.0 | 40.0–125.0 | Alert trigger threshold °C. Clears at threshold minus 2°C. |

# 9. Error Handling

| Failure | Detection | Response |
| --- | --- | --- |
| No ACK at boot | Wire.endTransmission() != 0 | _sensorOk=false. Log (MIRI_DEBUG). Report ERROR in info panel. |
| Read failure in loop | requestFrom() returns 0 | Increment error count. Keep last good value. Log after 5 consecutive failures. |
| Value out of range | tempC < -40 or > 150 | Discard. Log. Do not update MiriState. |

# 10. Open Questions

❓  Open Question: Is 2-second polling acceptable, or is a faster/slower rate needed?

❓  Open Question: Should the ALERT pin be wired to a GPIO in a future board revision for interrupt-driven detection?